Diffusion optics project

Goal- we will introduce multi-plane wavefront shaping (MWFS) that will achieve a perfect phase correction using an SLM.

To simulate the aberrations, we will generate a 3D tissue model and slice (average) it to M discrete planes, which will represent the static tissue. Each one of these planes will be marked using a capital letter, and the distance between them will be denoted as ϵ. The free space propagation between these planes will be denoted using a matrix Pϵ.

Therefore the light coming out of the tissue is represented as uout of tissue=BPϵAPϵuin in case we have only 2 aberration planes, and uin is the illumination field within the tissue.

The correction planes on the SLM will be represented using capital letters with tildes. Since the light is focusing gradually during correction, we can say it backpropagates a distances of -ϵ between correction planes.

So the corrected light uout is uout=P-ϵ*A@P-ϵ*B@B@Pϵ*A@Pϵ*uin, where * is convolution and @ is matrix multiplication. Since P-ϵ=Pϵ-1, if each correction plane will have X=X-1 we will achieve a full correction- uout=uin. Moreover, since the aberration and correction planes consist of only complex phases (e-jϕ, ϕ∈0,2π), we can have X=X*. Note that we stated only 2 planes for simplicity, but could be any M. The number of planes is derived from the total tissue width and the step size.

Dimensions- uin∈CN2x1 is a column stack vector of an NxN image
Pϵ,P-ϵ will be applied using a multiplication in the frequency domain, and not using 
propagation in free space.
A, B and all possible aberration/correction planes will be diagonal N2xN2 matrices, consisting of complex exponents in the diagonal and zeros otherwise.

Propagation will be done using the angular spectrum method- transform into frequency domain, multiply each freq component of u by a phase corresponding to the propagation distance, with relation to the wavelength. Therefore we won’t actually convolve, but instead transform -> multiply -> inverse transform.

Aberration planes- tissue_3d_generator.m returns both — mask_f0 (zero-mean fluctuation only) and mask_f (fluctuation + mean transmission mu). Define each aberration plane as A_k = exp(i·angle(mask_f(:,:,k))). Same will go for all aberration planes.

Input field- we work in grescale, complex values (single complex channel). Start with a 1.0 + 0i in the central pixel, simulating a single point source in the middle of the image.

Sim step 1

Set A=A*, B=B*, only 2 planes. Plot the absolute value of- the input field, the field after each aberration and the field after each correction

Propagation kernel used in step 1

For Pε and P−ε we use the standard (exact, non-paraxial) angular spectrum kernel: H(fx,fy)=exp(i·2π·(ε/λ)·√(1−(λfx)²−(λfy)²)) for propagating components where (λfx)²+(λfy)²<1, and H=0 for evanescent components where (λfx)²+(λfy)²≥1 (dropped, not propagated). λ=532nm (=0.532µm); ε and all spatial dimensions (x_max, x_stp, z_max, pixel pitch) are in µm. The alternative we are not using is the paraxial/Fresnel approximation, H(fx,fy)=exp(ikε)·exp(−iπλε(fx²+fy²)), which is cheaper but only valid for small propagation angles and does not exactly null evanescent components; at the fine sampling used here (~0.1µm pixels vs. λ=0.532µm) the exact kernel is preferred.